### Lab Program 9: IoT Data Processing with ESP32 + DHT22 + ThingSpeak (Wokwi Simulator)

This lab program covers your requirements using ThingSpeak for visualization and historical data storage (instead of a custom MQTT broker). It includes:

- Reading sensor data (temperature & humidity from DHT22)

- Data processing: Simple threshold-based alerts (e.g., high/low temperature)

- Alerts: Local LED + serial message (ThingSpeak can also show alerts via MATLAB analysis if needed)

- Secure communication: Uses HTTP POST over TLS – ThingSpeak supports secure connections

- Visualization: Real-time & historical charts on ThingSpeak dashboard

#### Step 1: Setup ThingSpeak Account (One-time)

- Create a free account at https://thingspeak.com

- Create a new channel with:

  - Field 1: Temperature (°C)

  - Field 2: Humidity (%)

  - (Optional) Field 3: Alert Status (0 = normal, 1 = alert)

- Note down:

  - Channel ID (e.g., 1234567)

  - Write API Key (e.g., XXXXXXXXXXXXXXXX)

#### Step 2: Create Wokwi Project

- Go to https://wokwi.com → New Project → ESP32

- Add components:

  - DHT22 sensor → connect data pin to GPIO 15

  - LED (for alerts) → connect to GPIO 2 (with resistor)

- Use Arduino framework

#### Step 3: Complete Arduino Code (ESP32)

C++

#include <WiFi.h>

#include <HTTPClient.h>        // For HTTP POST

#include <WiFiClientSecure.h>  // For HTTPS (secure)

#include <DHTesp.h>            // DHT library for ESP32

// WiFi (Wokwi default)

const char* ssid = "Wokwi-GUEST";

const char* password = "";

// ThingSpeak settings (replace with your own)

const char* server = "api.thingspeak.com";

const int   channelID = 1234567;              // YOUR CHANNEL ID

const char* writeAPIKey = "XXXXXXXXXXXXXXXX"; // YOUR WRITE API KEY

// Sensor & LED pins

#define DHT_PIN       15

#define LED_ALERT_PIN 2

// Thresholds

#define TEMP_HIGH_THRESHOLD  30.0

#define TEMP_LOW_THRESHOLD   18.0

#define HUM_HIGH_THRESHOLD   80.0

DHTesp dht;

WiFiClientSecure client;  // Secure client

void setup() {

Serial.begin(115200);

pinMode(LED_ALERT_PIN, OUTPUT);

digitalWrite(LED_ALERT_PIN, LOW);

// Initialize DHT22

dht.setup(DHT_PIN, DHTesp::DHT22);

// Connect to WiFi

WiFi.begin(ssid, password);

Serial.print("Connecting to WiFi...");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi connected");

Serial.print("IP address: ");

Serial.println(WiFi.localIP());

// Allow insecure certs for simulation (ThingSpeak certs are valid, but Wokwi sometimes needs this)

client.setInsecure();

}

void loop() {

static unsigned long lastSend = 0;

if (millis() - lastSend < 20000) {  // ThingSpeak free tier: min 15-20s interval

delay(100);

return;

}

lastSend = millis();

// Read sensor

TempAndHumidity data = dht.getTempAndHumidity();

if (dht.getStatus() != DHTesp::ERROR_NONE) {

Serial.println("DHT read error!");

return;

}

float temp = data.temperature;

float hum = data.humidity;

Serial.printf("Temp: %.1f °C | Humidity: %.1f %%\n", temp, hum);

// Process data: Check thresholds & generate alert

int alert = 0;

String alertMsg = "Normal";

digitalWrite(LED_ALERT_PIN, LOW);

if (temp > TEMP_HIGH_THRESHOLD) {

alert = 1;

alertMsg = "HIGH TEMP";

digitalWrite(LED_ALERT_PIN, HIGH);

} else if (temp < TEMP_LOW_THRESHOLD) {

alert = 1;

alertMsg = "LOW TEMP";

digitalWrite(LED_ALERT_PIN, HIGH);

} else if (hum > HUM_HIGH_THRESHOLD) {

alert = 1;

alertMsg = "HIGH HUMIDITY";

digitalWrite(LED_ALERT_PIN, HIGH);

}

Serial.println("Alert: " + alertMsg);

// Send to ThingSpeak via HTTPS POST

if (WiFi.status() == WL_CONNECTED) {

HTTPClient http;

String url = "https://" + String(server) + "/update?api_key=" + writeAPIKey +

"&field1=" + String(temp, 1) +

"&field2=" + String(hum, 1) +

"&field3=" + String(alert);

http.begin(client, url);  // Secure connection

int httpCode = http.GET();  // Actually a POST under the hood for update

if (httpCode > 0) {

Serial.printf("ThingSpeak Response: %d\n", httpCode);

if (httpCode == 200) {

Serial.println("Data sent successfully!");

}

} else {

Serial.println("HTTP request failed");

}

http.end();

} else {

Serial.println("WiFi lost");

}

delay(100);  // Small delay for simulation stability

}

#### Step 4: How to Test in Wokwi

- Paste the code into the editor

- Replace channelID and writeAPIKey with your real values

- Click Start Simulation

- Click on DHT22 sensor → adjust temperature/humidity sliders

- Watch Serial Monitor for readings and alerts

- LED on GPIO 2 lights up on alert

- Go to your ThingSpeak channel → Private View or Public View:

  - Real-time charts for temperature & humidity

  - Historical data stored automatically

  - Use MATLAB Visualization in ThingSpeak for alerts (e.g., highlight when field3 == 1)

Lab Program 10: IoT Data Processing with Raspberry Pi Pico W + HC-SR04 Ultrasonic Sensor + ThingSpeak (Wokwi Simulator)

This lab program adapts the requirements for Raspberry Pi Pico W using ThingSpeak for visualization and historical data storage. It includes:

- Reading sensor data (distance from HC-SR04)

- Data processing: Simple threshold-based alerts (e.g., object too close)

- Alerts: Local LED + console message (ThingSpeak can also show alerts via MATLAB analysis if needed)

- Secure communication: Uses HTTP GET over TLS – ThingSpeak supports secure connections

- Visualization: Real-time & historical charts on ThingSpeak dashboard

Step 1: Setup ThingSpeak Account (One-time)

- Create a free account at https://thingspeak.com

- Create a new channel with:

  - Field 1: Distance (cm)

  - (Optional) Field 2: Alert Status (0 = normal, 1 = alert)

- Note down:

  - Channel ID (e.g., 1234567)

  - Write API Key (e.g., XXXXXXXXXXXXXXXX)

Step 2: Create Wokwi Project

- Go to https://wokwi.com → New Project → Raspberry Pi Pico W

- Add components:

  - HC-SR04 ultrasonic sensor → connect Trigger to GP15, Echo to GP14

  - LED (for alerts) → connect to GP13 (with resistor)

- Use MicroPython framework

Step 3: Complete MicroPython Code (Raspberry Pi Pico W)

Python

import network

import urequests

import time

import machine

# WiFi (Wokwi default)

ssid = "Wokwi-GUEST"

password = ""

# ThingSpeak settings (replace with your own)

server = "api.thingspeak.com"

channel_id = 1234567  # YOUR CHANNEL ID

write_api_key = "XXXXXXXXXXXXXXXX"  # YOUR WRITE API KEY

# Sensor & LED pins

trigger_pin = machine.Pin(15, machine.Pin.OUT)

echo_pin = machine.Pin(14, machine.Pin.IN)

led_alert_pin = machine.Pin(13, machine.Pin.OUT)

# Threshold

DISTANCE_CLOSE_THRESHOLD = 10.0  # cm

def connect_wifi():

wlan = network.WLAN(network.STA_IF)

wlan.active(True)

wlan.connect(ssid, password)

print("Connecting to WiFi...")

while not wlan.isconnected():

time.sleep(0.5)

print(".")

print("\nWiFi connected")

print("IP address:", wlan.ifconfig()[0])

def measure_distance():

trigger_pin.off()

time.sleep_us(2)

trigger_pin.on()

time.sleep_us(10)

trigger_pin.off()

while echo_pin.value() == 0:

signal_off = time.ticks_us()

while echo_pin.value() == 1:

signal_on = time.ticks_us()

time_passed = signal_on - signal_off

distance = (time_passed * 0.0343) / 2  # Speed of sound: 343 m/s, divide by 2 for round trip

return distance

connect_wifi()

last_send = 0

while True:

if time.time() - last_send < 20:  # ThingSpeak free tier: min 15-20s interval

time.sleep(1)

continue

last_send = time.time()

# Read sensor

distance = measure_distance()

print(f"Distance: {distance:.1f} cm")

# Process data: Check threshold & generate alert

alert = 0

alert_msg = "Normal"

led_alert_pin.off()

if distance < DISTANCE_CLOSE_THRESHOLD:

alert = 1

alert_msg = "OBJECT TOO CLOSE"

led_alert_pin.on()

print("Alert:", alert_msg)

# Send to ThingSpeak via HTTPS GET

url = f"https://{server}/update?api_key={write_api_key}&field1={distance:.1f}&field2={alert}"

try:

response = urequests.get(url)

print("ThingSpeak Response:", response.status_code)

if response.status_code == 200:

print("Data sent successfully!")

response.close()

except Exception as e:

print("HTTP request failed:", e)

time.sleep(1)  # Small delay for stability

Step 4: How to Test in Wokwi

- Paste the code into the editor

- Replace channel_id and write_api_key with your real values

- Click Start Simulation

- Click on HC-SR04 sensor → adjust the distance slider

- Watch the console (via Serial Monitor) for readings and alerts

- LED on GP13 lights up on alert

- Go to your ThingSpeak channel → Private View or Public View:

  - Real-time charts for distance

  - Historical data stored automatically

  - Use MATLAB Visualization in ThingSpeak for alerts (e.g., highlight when field2 == 1)